# Aircraft configurations:

# overall config.:

look at wingspan & weight!!

LSA:

at the very extreme, too risky. (weight and span)

single propeller GA aircraft:

very safe (perfect span and lots of leeway for weight)

sailplane:

span>> & not enough weight

all others:

overkill, huge payload

# wing location:

| coeff |  | low wing | mid wing | high wing |
| --- | --- | --- | --- | --- |
| 3 | fuel feeding system | 1 (need pump) | 2 | 3 (gravity fed) |
| 6 | ground effect | 3 | 2 | 1 |
| 4 | best for heavy transport | 3 | 1 | 2 (need anhedral) |
| 2 | least fairring challenges | 2 | 3 | 1 |
| 5 | maneuvrability | 2 (need dihedral) | 3 | 1 |
| 1 | aesthetics ;) | 1 | 3 | 2 |
| 21 |  | 48 | 46 | 32 |

→ low wing

→ wing dihedral for maneuvrability

→ contributes to C_l(beta)

# # of wings:

sesquiplane: for crop dusting cus spreads the fluid with 4 vortices if we use water

triplane: too complex (novelty)

| coeff |  | monoplane | biplane |
| --- | --- | --- | --- |
| 2 | efficiency | 2 | 1 |
| 3 | wingspan | 1 | 2 |
| 1 | maneuverability | 1 | 2 |
| 4 | simplicity | 2 | 1 |
| 10 |  | 16 | 14 |

→ monoplane

BUT!!! could do biplane if you’re hyped for it

# wing structure:

| coeff |  | cantilever | strut-braced |
| --- | --- | --- | --- |
| 1 | aesthetic | 2 | 1 |
| 3 | novelty | 2 | 1 |
| 2 | efficiency | 2 | 1 |
| 4 | feasibility (material & manufacturing) | 1 | 2 |
| 10 |  | 16 | 10 |

→ cantilever wing

# Propeller configuration:

| coeff |  | tractor | pusher |
| --- | --- | --- | --- |
| 1 | observation ability | 1 | 2 |
| 2 | novelty | 2 | 1 |
| 3 | propeller efficiency | 2 | 1 |
| 6 |  | 11 | 7 |

→ tractor

# engine placement:

simply taking into account the most conventional placement and limiting the pitching moment by having delta_z ≈ 0.

→ F

→ can align T vector vector c.g. to limit pitching moment

# landing gear config.:

only look at 2 most common: tail dragger and tricycle for simplicity

| coeff |  | tail dragger | tricycle |
| --- | --- | --- | --- |
| 2 | drag | 2 | 1 |
| 3 | weight | 2 | 1 |
| 5 | most adaptable for runways (AOA) | 2 | 1 |
| 4 | stability (risk for ground loop) | 1 | 2 |
| 1 | novelty | 1 | 2 |
| 15 |  | 25 | 20 |

→ taildragger

# Tail config.:

→ conventional tail

because there’s no  special requirements, and nothing preventing from using it (simplicity and effectiveness)