Fire Extinguishing Devices Trade-Off

| Technology Suggestions Criteria | Red Powder/ Dry Chemical (“Fire Retardants”) | Extinguishing Grenade/ Dry Powder Bomb | Water |
| --- | --- | --- | --- |
| Area Efficiency per mass | Covers a high area per kg 4.21 m² per kg [1] | 30 kg dry powder bomb diffuses to blanket an area between 130 and 700 square meters: 4.3 to 23.3 m² per kg [2] | Poor coverage 0.01 to 0.2 m² per kg [3] |
| Area Efficiency per flight minute | ~350 to 400 m2 per minute [4] | ~30 m2 per minute [2] | Heavy mass limits discharge time ~210 to 310 m2 per minute [5] |
| Precision Tolerance | Does not have to be precise as broad cloud covers broad areas | Precise kinetic delivery, does not succumb so easily to wind but the trajectory upon impact must be precise to target fire correctly. can cover much area if exploding takes place a few meters before hitting, one-shot impact, high localized area coverage, and does not depend too much on wind conditions to reach target. Better control on where it falls compared to other solutions. | Requires direct continuous hits |
| Total Extinguishing and risk of reignition | Rapid chemical knockdown, creates long-term barriers Poor thermal cooling => high risk of reignition | Extremely effective on knocking canopy flames. The kinetic blast of a dry powder bomb knocks out active flames instantly by starving them of oxygen. However, it does nothing to lower the core temperature of thick wood => very high risk of reignition | Prevents reignition by thermally cooling fuel (when not evaporating too much…) |
| Suitability for outdoors (wind) | Wind assists the overall distribution which is good | Yes. Wind changes its trajectory the least. | Yes |
| Environmental impact | Ammonium polyphosphates: Toxic, especially for aquatic ecosystems (fish) and vegetation | ABC Monoammonium Phosphate: Very toxic, especially for aquatic ecosystems (fish), vegetation and terrestrial wildlife | No |
| Cost | Moderate | High | Low |
| Availability/ Accessibility | Restricted (Requires advanced manufacturing and mixing bases) | Highly Accessible (Stored easily in local fire stations) | Theoretically infinite and immediate (but depends on location and up to water availability) |

Weighing factors legend:

Importance of criteria: 1-7

Marks: 1-3

| Technology Suggestions Criteria | Red Powder/ Dry Chemical (“Fire Retardants”) | Extinguishing Grenade/ Dry Powder Bomb | Water |
| --- | --- | --- | --- |
| Area Efficiency per mass (7) | 2 | 3 | 1 |
| Area Efficiency per flight minute (6) | 3 | 1 | 2 |
| Precision Tolerance (3) | 2 | 3 | 1 |
| Total Extinguishing and risk of reignition (3) | 1 | 2 | 3 |
| Suitability for outdoors (Type A Fires) (5) | 3 | 3 | 3 |
| Environmental impact (2) | 1 | 1 | 3 |
| Cost (1) | 2 | 1 | 3 |
| Availability/ Accessibility (2) | 1 | 2 | 3 |
| TOTAL: | 62 | 64 | 61 |

Red Powder Examples:

- https://www.perimeter-solutions.com/product/phos-chek-259-fx-2/

- https://xpyro.fi/ (more eco-friendly)

Dry Powder Bomb Examples:

- https://jinghongdrone.com/fire-extinguishing-bomb-deployment-device (bombs (23 kg each) with 20 kg extinguishing agent per unit, providing coverage of ≥ 50 m² and safe wireless deployment)

- https://www.youtube.com/shorts/bTtY0pKd4pU

- How it works: https://salgrom.fi/en/how-does-impulse-dry-powder-fire-suppression-work/

To be noted: Water deployment remains indispensable at some stage of suppression because it absorbs residual thermal energy to prevent reignition. While these tactics complement one another, payload-limited UAVs should prioritize precision chemical retardants or dry-powder bomb strategies rather than bulk water delivery, which requires a much heavier cargo capacity.

References

[1] https://www.google.com/url?sa=i&source=web&rct=j&url=/goto?url%3DCAESlgEB6zswFaW6jig_nAzX5a_wuSaun9QoBNS2FJNdq4DhdCqCsjF2YeGIPwS30PeL7lgwNpzXbBMquxrUHsJ1aPgX7LqV9FkTJmAQVq-u6UTnTqVSLL_fguXw6LhQijW65jju9XBnDlppdOvEpJsm8bgwboF16mUq8fn1g9E8_LpZjDJF-7ScNL422HSru0BzK4vkxFvbdXI&ved=2ahUKEwiqssbbkIOXAxVdHhAIHW0QHMQQqYcPegYIAAgnEAI&opi=89978449&cd&psig=AOvVaw0KnjIk9wtdBAUFHfSz9hOf&ust=1790198896298000

[2]

https://jinghongdrone.com/uav-for-fire-fighting-key-technologies-and-features-explained

[3]

https://wmpllc.org/ojs/index.php/jem/article/view/2689

[4]

https://www.fs.usda.gov/t-d/pubs/pdfpubs/pdf00572851/pdf00572851_72dpi.pdf

[5]

https://en.wikipedia.org/wiki/Canadair_CL-415

To do:

- Takeoff distance

- Climb rate

- Cruise speed

- Altitude

- Range

- μ (surface)

- density/altitude: Sicherheitshöhe? => Inspiration from existing red powder planes

- Mission profile (report?, how big is the fleet? , T-O/Cruise/Landing)

- payload in numbers

- operating environment

- Regulatory framework ()

- Station plan () & logistic, permanent stationiert/ Armee

- Range: circles

- Take-Off Distance

- Payload Weight (200 kg?)

- How much km2 per bomb?

Niklas: Fläche im Bezug auf spezifische Beispiele

Sego : Parameters,